|  | Pre-Delivery Inspection Form | Etnyre Chipspreader Machines Fixed - Variable Width |
| --- | --- | --- |
| 1333 South Daysville Road Oregon, IL 61061 Phone: 815-732-2116 Fax: 800-995-2116 Email: sales@etnyre.com | Customer: Date: Model: Unit Serial #: Dealer: Branch: Inspected by [print] [sign] |  |

Instructions: Check a box on every line and make sure to comment on any defects of issues found. Enter stock unit on Customer line if unit is not sold yet. Return Completed Pre-Delivery Inspection Form with all pages to Warranty@etnyre.com with subject line “[Unit Serial #] PDI Report”

GENERAL:

| Item | OK | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Is machine free of damage? | ☐ | ☐ | ☐ | ☐ |  |
| Correct machine specifications to order. | ☐ | ☐ | ☐ | ☐ |  |
| Correct hopper size & style. | ☐ | ☐ | ☐ | ☐ |  |

APPEARANCE:

| Item | OK | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Paint. | ☐ | ☐ | ☐ | ☐ |  |
| Chain guards. | ☐ | ☐ | ☐ | ☐ |  |
| Safety railings. | ☐ | ☐ | ☐ | ☐ |  |
| Decals. | ☐ | ☐ | ☐ | ☐ |  |
| Anti-skid tape (deck). | ☐ | ☐ | ☐ | ☐ |  |
| Flashing. | ☐ | ☐ | ☐ | ☐ |  |
| Front hopper gates. | ☐ | ☐ | ☐ | ☐ |  |
| Rear hopper gates. | ☐ | ☐ | ☐ | ☐ |  |

ENGINE SYSTEMS:

| Item | OK | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Oil levels. | ☐ | ☐ | ☐ | ☐ |  |
| Oil leaks. | ☐ | ☐ | ☐ | ☐ |  |
| Coolant levels. | ☐ | ☐ | ☐ | ☐ |  |
| Coolant leaks. | ☐ | ☐ | ☐ | ☐ |  |
| Radiator condition. | ☐ | ☐ | ☐ | ☐ |  |
| Battery condition. | ☐ | ☐ | ☐ | ☐ |  |
| Battery fluid level (if able to check). | ☐ | ☐ | ☐ | ☐ |  |
| Battery terminals/check tightness of connections. | ☐ | ☐ | ☐ | ☐ |  |
| Starter operation. | ☐ | ☐ | ☐ | ☐ |  |
| Alternator operations/voltage output. | ☐ | ☐ | ☐ | ☐ |  |
| Fan belt condition. | ☐ | ☐ | ☐ | ☐ |  |
| Engine guards are in place and secure. | ☐ | ☐ | ☐ | ☐ |  |

AIR SYSTEM:

| Item | OK | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Air shut off valve on at air oiler ASM. | ☐ | ☐ | ☐ | ☐ |  |
| Oil level in oiler to fill line inside cup. (10w non-detergent oil or equivalent) | ☐ | ☐ | ☐ | ☐ |  |
| Air leaks. | ☐ | ☐ | ☐ | ☐ |  |
| Air pressure at regulator on air oiler ASM. set at 85-90psi. | ☐ | ☐ | ☐ | ☐ |  |

HYDRAULIC SYSTEM:

| Item | OK | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Hydraulic tank oil level, (no more than 1/2 on sight glass) do not overfill, ISO 46 hydraulic oil. | ☐ | ☐ | ☐ | ☐ |  |
| Pumps | ☐ | ☐ | ☐ | ☐ |  |
| Motors. | ☐ | ☐ | ☐ | ☐ |  |
| Manifold blocks. | ☐ | ☐ | ☐ | ☐ |  |
| Cylinders. | ☐ | ☐ | ☐ | ☐ |  |
| Hydraulic cooler (part of radiator ASM.) | ☐ | ☐ | ☐ | ☐ |  |
| Fan drive motor. | ☐ | ☐ | ☐ | ☐ |  |
| Reversing fan (optional) only operates if fan drive is active. | ☐ | ☐ | ☐ | ☐ |  |
| Hoses/fittings/connections. | ☐ | ☐ | ☐ | ☐ |  |
| House routing & fastening. | ☐ | ☐ | ☐ | ☐ |  |
| Filters & indicators. | ☐ | ☐ | ☐ | ☐ |  |

OPERATOR’S STATION SWITCHES & CONTROLS:

* Be sure to make note of any specific issues or problems during your testing. Record your notes in the adjacent “Comments” section.

| Item | OK | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Test all main operator panel switches and controls for proper operation. * | ☐ | ☐ | ☐ | ☐ |  |
| Test all front control panel switches and controls for proper operation. * | ☐ | ☐ | ☐ | ☐ |  |
| Parking brake operation. | ☐ | ☐ | ☐ | ☐ |  |
| Service brake operation. | ☐ | ☐ | ☐ | ☐ |  |
| Hydraulic drive pump - FWD. & REV. | ☐ | ☐ | ☐ | ☐ |  |
| Steering operation - L. & R. | ☐ | ☐ | ☐ | ☐ |  |
| Traction control boost (200FPM with boost switch on.) | ☐ | ☐ | ☐ | ☐ |  |
| Traction boost hitch (if equipped.) | ☐ | ☐ | ☐ | ☐ |  |
| Speed control goes to the desired FPM. | ☐ | ☐ | ☐ | ☐ |  |
| Gate & spread roll operation (top of spread roll turns towards gate openings. | ☐ | ☐ | ☐ | ☐ |  |

OPERATOR’S STATION SWITCHES & CONTROLS (cont’d):

| Item | OK | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Conveyor auto - conveyor belts turn in correct direction (towards front hoppers.) | ☐ | ☐ | ☐ | ☐ |  |
| Conveyor manual - both panels. | ☐ | ☐ | ☐ | ☐ |  |
| Conveyor appearance & condition. | ☐ | ☐ | ☐ | ☐ |  |
| Conveyor auto switches - sonic sensor adjust/calibrate to approx. 9”-11” out from aggregate. | ☐ | ☐ | ☐ | ☐ |  |
| Auger auto - auger flighting spirals outwards toward outer end of hoppers. | ☐ | ☐ | ☐ | ☐ |  |
| Auger manual - both panels. | ☐ | ☐ | ☐ | ☐ |  |
| Vibrators (if equipped.) | ☐ | ☐ | ☐ | ☐ |  |
| Electrical wiring/routing/fastening. | ☐ | ☐ | ☐ | ☐ |  |

ENGINE MONITORING PANEL:

| Item | OK | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Scroll through screens and test monitor communication. | ☐ | ☐ | ☐ | ☐ |  |
| Verify DEF fluid level. | ☐ | ☐ | ☐ | ☐ |  |
| Make sure there are no active codes or warnings. | ☐ | ☐ | ☐ | ☐ |  |

TIRES & SERVICE:

| Item | OK | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Tire condition. | ☐ | ☐ | ☐ | ☐ |  |
| Wheels & lug nuts condition. | ☐ | ☐ | ☐ | ☐ |  |
| Tire pressure.*†¥ | ☐ | ☐ | ☐ | ☐ |  |
| Hitch positioning. (Can be relocated to suit the customer’s need to attach trucks.) | ☐ | ☐ | ☐ | ☐ |  |
| Grease. (If needed.) ˆ | ☐ | ☐ | ☐ | ☐ |  |

* - STD 2WD & 4WD Unit Tire Pressure = 65-70psi

† - VW 2WD & 4WD Unit Tire Pressure = 75-80psi.

ˆ - Grease requirement is molub - alloy spec; extreme pressure lubes are not recommended on air gates or hitch pivot points.

TOOL KIT ITEMS:

| Item | OK |
| --- | --- |
| 2 - Canvas tarps (Var. Width.) |  |
| 1 - Canvas tarp (Fixed Width.) |  |
| 1 - Scale. |  |
| 1 - Hitch rod. |  |
| 1 - Switch location guide. |  |
| 1 - Manual holder. |  |
| 1 - Operations manual. |  |
| 1 - Parts manual. |  |
| 1 - Cummins engine registration. |  |
| 1 - Delivery Start-Up Report Form. |  |